BATCH102.CSV

- Date, Time, Sensor ID, Bed ID, Soil Moisture, Soil Temp

- Probably a wireless soil-sensor network with a central data logger, used for irrigation monitoring. Evidence: Ten fixed sensor IDs. Moisture and temperature measured at the same time and location as the capacitance probes. All sensors report at the same timestamp every 2 hours, indicating automated, synchronized polling

- Date + Time + SensorID.

- Date: 07/15/2026, Time: 12:00 AM, SensorID: SN-4821, BED ID: BED-1, Soil Moisture: 27.3, Soil Temp: 82.8

- 120

- Samples every 2 hours, reading all 10 sensors together in the same order each time.

One raw reading per sensor per cycle, no averaging, battery, signal, or error fields.

- Advantages

Plain, generic, & human-readable text, opens in any spreadsheet or program. Space-saving and simple to mount with labeled sensor and bed IDs. Grouping and filtering are easy to do thanks to the long format.

Disadvantages

Date and time are split, with no time zone, also MM/DD/YYYY is easy to misread. Units are implicit, and there is no metadata.  No unique row key or validation. The 2-hour interval can miss short events, so watering times are only known within a 2-hour window.

FEED102.XML

1. Air Temp, Humidity, Windspeed

2.  A small automatic weather station. Evidence:

The root element is Station with id "WS-01", so one named device.

The three measurements are standard weather-station sensors.

Readings arrive exactly every 10 minutes, all day, which points to automated logging.

3. Date and time

4. <observation date="15/07/2026" time="00:00">

<airTemperature unit="C">27.5</airTemperature>

<humidity unit="percent">74</humidity>

<windSpeed unit="mph">3.7</windSpeed>

5. 144 observations

6. How does the device collect and store data?

Samples every 10 minutes, from 00:00 to 23:50, with no missed intervals. Records all three measurements together in every observation, in the same order. Stores raw point readings only, no averages, gusts, wind direction, rainfall, battery, or error flags. Mixes unit systems: temperature in metric (°C) but wind in imperial (mph). Stores values with fixed precision: temperature and wind to 1 decimal, humidity as a whole number.

7.

Advantages

Self-describing, tag names and unit attributes explain every value.

Hierarchical and extensible, new sensors can be added without breaking existing structure.

Machine-readable and validatable , and widely supported.

Station ID is stored once, not repeated on every record.

Disadvantages

Repeated tags and unit attributes make the file much larger than the equivalent CSV. Dates and times are plain text with no timezone, and DD/MM/YYYY is easy to confuse with the US format. Units repeated on every reading are redundant, and the mixed metric/imperial units invite conversion mistakes. No unique record ID, and no metadata such as station location, sensor calibration, or quality flags.

STREAM102.JSON

1. An irrigation log

2. A multi-zone irrigation controller or pump station with a flow meter. Evidence:

A pumpStatus field with on/off states, plus a flow rate in gallons per minute. Zones run one after another, never overlapping: zone 1, then 2, then 3. The "stream" in the filename suggests an event feed from the controller.

3. Date, time and zone

4. "date": "07/15/2026",

"time": "5:00 AM",

"zone": 1,

"flowRateGpm": 3.57,

"pumpStatus": "on"

5. 38 entries

6. Logs only while irrigating, not on a fixed 24-hour schedule, so the file is event-driven. Samples every 5 minutes during a run, then writes one "off" entry with flow 0.0 about 5 minutes after the last reading. The first reading of each run is lower and then rises, reflecting pump or line pressure ramp-up.

7.

Advantages

Typed values with no parsing guesswork for flow or zone. Self-describing: key names label every value, and records can gain new fields without breaking old ones. Lightweight and native to web and programming tools, easy to stream or append. Handles irregular, event-based logging naturally, since no row is wasted on idle time.

Disadvantages

Keys repeat on every record, so it is larger than a CSV of the same data. Less convenient for spreadsheets and quick eyeballing, the whole array must be loaded or parsed to analyze. Redundant fields, pumpStatus duplicates what flow = 0.0 already shows. Date and time are text with no timezone and run start/stop and totals must be inferred.